Workshop: Next generation computing using FPGA’s

Exercise 1 Creating a logic module:

- Login to your hosted workstation. Open Vivado:

- Create a new Project using “Create Project”. Name the project “logicproject”

- Leave the default setting RTL Project

- Set the Target language on VHDL.

- Create a new vhdl file using the “Create File” button called logic

- Leave the default settings and go the next screen

- We will not be adding any constraints yet.

- Use as part a pre-made board (select by double-click the “Boards option)”. The board we will be using is the Arty Z7-20

- At define module, add 3 ports; a,b, and c using the +. Mark a and b as in(puts), c as out(put).

- Run the simulation using the “Run Simulation” -> ”Run Behavioral Simulation” on the left. Look at the functional simulation. U means “Uninitialized”.

- In scope, click the logic (vhdl module). This should open the vhdl specification.

- Add the following vhdl behavior: “c <= a OR b;”

- Rightclick a and force a 100ns clock on signal a, Edge values 1 and 0.

- Also add a clock to B and force a 33ns clock on signal a, Edge values 1 and 0.

- Rerun the simulation using the restart button (|<) and run for 10us (>(t)). If you need to find the black screen (waveform), its under Window -> Waveform

- Test around with multiple gates. Try to build a half adder yourself.

- Also try the following: “c <= a when a = ‘1’ else b;”

Exercise 2: Clocks

- Add a clk input signal to the port. Your VHDL should look like:

- Replace the “c <= …” by a process like this:

- You now need to refresh the simulator to pick this up. Stop simulation mode by using the X in the blue corner. And start the simulator again (hint: exercise 1).

- If you force a clock on clk (21ns works well). You will see that the output is more synchronized with the clk clock now:

- Zoom in on the 0ns. Can you explain what is happening there ?

- Try using “rising_edge(clk) or falling_edge(clk)”. What behavior do you expect ?

Exercise 3: Constraints

- “Add sources” to the constraint “constrs_1”, Add a new constraint file “logicboard”

- Add to logicboard.xdc  the content of: https://github.com/Digilent/Arty-Z7/blob/master/Resources/XDC/ArtyZ7_20Master.xdc Uncomment the; clock, switches, and a led. Replace “sw[0]” by ‘a’ and “sw[1]” to ‘b’. This will make the signals ‘a’ an ‘b’  be connected to the buttons. Replace “led[0]” by ‘c’. Comment out the rest.

- Make sure you only are using “rising_edge(clk)”. Start the synthesis, perform the implementation and open the implemented design:

… Ok, Ok, Ok

- You should see the physical chip design. In the netlist, you can zoom to a connection:

- Generate the bitstream. This will generate a firmware which can be flashed to the board. Flash this your own device or ask the tutor to show you a flashed one.

- We are going to make the calculation a bit more exiting:

- Simulate to check its behavior. Generate the design again and see that design becomes more and more complex.

- Create the signals of  http://burnt-traces.com/?p=246 and see that the design starts becoming more complex and complex.

- Congratulations you now can use FPGA’s to create projects